Goal: Low cost fly-by-wire control system.

Main issues we want to avoid:

- Step Loss: Sending commands before the hardware has had a chance to process the last command.

- Syncronization Loss: The motor is physically moved without the computer knowing about it.

- Excess Motor movement: We want a nice smooth movement from the motor with out unneccessary steps and 'jitters'.

- Irregular duty cycle: We want a duty cycle close to 50% otherwise there is a chance of vibrations, also the motor efficientcy will be impaired.

Equipment used:

- Centent CN0162 stepper motor driver.

- Tormach Size 34 - 640 oz in stepper motor.

- Logitech WingMan USB joystick.

- VIA EPIA-5000 Eden 533MHz Fanless CPU mini-ITX

- 1.5GB Laptop harddrive.

Software used:

- Redhat Linux 9.0

- Joystick Wrapper Library libjsw (for joystick input)

- Mini-XML parser libmxml (for configuration file)

- gcc/gdb/make (for compiling, debuging, etc).

- ssh (For communication with computer).

- Subctrl ( the software devloped )

Result Summary:

- Stepper motor driver gets hot after about 15 minutes.

- Stepper motor takes 1 second to move 360degrees (2000 steps per second).

- No parallel port "step loss" was detected while testing.

- Powering opto-isolators from same power source as motor current was not possible.

- Changing directions more than once in 50 steps can result in vibrations

- vibrations lead to syncronization loss (the motor position jumps).

- Cheap joysticks sometimes give 'flickering' readings which oscilate between two values.

Conclusions:

Heat problems:

- lowering the stand by current value/resistor

- adding a heat sink

Motor Speed:

- limited by Parallel port communications rate

- possible to get over 3500 steps per second

- higher the speed higher the step loss risk

- Updating the parallel port too fast will result in lost steps

- Other Solutions: Scientic Sampling PCI boards, or USB Personal Measurement Devices[1].

Lost steps:

- the pilot part of the feed back loop

- Handling inconsitancies = possible step lost

- a reset button in the event of step lost

- Alterntive solution: micro switch (or reed switch) at a know point

Conclusions (continued):

Opto-isolators need clean power:

- Motor's power source caused feedback problems

- Use seperate battery solved the problems

Motor vibrations:

- Changing motor directions rapidly can cause vibrations

- Vibrations can cause motor to jump (syncronization lost)

- Mounting material can effect vibrations and harmonics

- Limit to one direction change per give steps (configurable)

- still possible for harmonic build up causing vibrations

Joystick Oscilations:

- Joystick input value oscilating

- Oscilations cause vibrations in motor movement

- At best causes unneccessary motor movement

- Set a timeout for joystick axis which is oscilating

- Higher (than timeout) harmonics still possible

[1] - http://www.measurementcomputing.com/cbicatalog/cbiproduct_new.asp?dept_id=352&pf_id=1535